1. ELECTRIC CURRENT (x)

- Define potential difference? Why do voltmeters always connected in parallel across the points between which the P.D. is to be measured?

- Explain Ohm’s law and verify it? Show how is it possible when resistance is doubled the current gets halved.

- Explain the factors on which the resistance of a conductor depends?

- Why does resistivity of an alloy is generally higher than that of its constituent’s metals?

- Resistance of a metal wire of length 1m is 26Ω at 200 C. If the diameter o the wire is 0.3 mm, what will be the resistivity of the metal at the temperature?

- A 4Ω resistance wire is doubled on it. Calculate the new resistance of the wire.

- How do we calculate total resistance when it is connected in series and parallel and why?

- In fig R1 = 10Ω, R2 = 40Ω, R3 = 30Ω,

R4 = 20Ω, R5 = 60Ω and a 12V battery is

connected to the arrangement. Calculate

the total resistance in the circuit and (b)

the total current flowing in the circuit.

- What do you understand by heating effect of electric current? Derive its formula?

- 100 J of heat are produced each second in a 4Ω resistance. Find the potential difference across the resistor.

- How do you calculate electric power of a component?

- An electric bulb rated 400w operates 8hr a day. What is the cost of energy to operate it for 30days at Rs. 3.00 per kw.h.

- Calculate the heat generated while transferring 96000 coulomb of charge in one hour through a P.D. of 50V.

- A battery of 9V is connected in series with resistors of 0.2 Ω, 0.3 Ω, 0.4 Ω

0.5Ω and 12 Ω. Respectively how much current would flow through the 12Ω resistance?

- Which uses more energy 250 WTV set in 1hr or 1200 W toaster in 10min?